| HL-LHC: Quality Manufacturing and Inspection Plan – Cable Fabrication |  |  |  |  |
| --- | --- | --- | --- | --- |
| Prepared by: I. Pong, J. Fleiter Date: 02/01/2018. Verified by: J. Blowers, G. Ambrosio, A. Ballarino, I. Bejar Alonso, P. Ferracin 04/04/2018 Date: DD/MM/20YY, DD/MM/20YY Date:. Approved by: G. Apollinari, E. Todesco, A. Devred Date: 06/04/2018 Date:. | Project: HL-LHC | Supplier: LBNL & FNAL | Item Eq. Code: MQXFAC | Asset Code (LHC Part Identifier): HCMQXFAC002-LBNNNNNN |
|  | Work Package: WP3 | Client: CERN | Item description: MQXFA Cable | EDMS Report No: |

| No | ACTIVITY / OPÉRATION | APPL. STANDARDS / NORMES APPL. | APPLICABLE DOCUMENTS / DOCUMENTS APPLICABLES | REV. DOC. | INSPECTION / CONTRÔLE |  |  |  |  |  |  |  | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SUPPLIER |  | CLIENT |  | 3RD PARTY |  | INSPECTION REPORT | REV. DOC. |  |
|  |  |  |  |  | Code | Signature/Date | Code | Signature/Date | Code | Signature/Date |  |  |  |
| 1.0 | Approval for strands and core usage (*) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | Approve strands for cabling |  | US HiLumi Doc 40 |  | H |  |  |  |  |  |  |  |  |
| 1.2 | Approve core for cabling |  | US HiLumi Doc 40 |  | H |  |  |  |  |  |  |  |  |
| 2.0 | Cabling map |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | Select wire spools for cabling (strand mapping) |  | US HiLumi Doc 74 |  |  |  |  |  |  |  |  |  |  |
| 2.2 | Select tooling and production parameters |  | Cable Fab Proc SU-1004-0963, Vector Traveler #464234 |  | N |  |  |  |  |  |  |  |  |
| 2.3 | Issue work order and respool log |  | Cable Fab Proc SU-1004-0963 |  | R |  |  |  |  |  |  |  |  |
| 3.0 | Respooling |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Respool strands |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 3.2 | This box is Intentionally left blank |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.0 | Preparation of the cabling machine |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Perform general inspection and preparation of cabling machine |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2 | Load spools and brakes according to cable map |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 4.3 | Install / verify installation of rollers on Turkshead |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 4.4 | Install / verify installation of mandrel |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 4.5 | Align Turkshead and mandrel |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 4.6 | Install stainless-steel core |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 4.7 | Barcode scan the spool map |  | Cable Fab Proc SU-1004-0963, Vector Traveler #464234 |  | N |  |  |  |  |  |  |  |  |
| 4.8 | String and measure tension of all strands |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 5.0 | Production of startup sample |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Calibrate CME |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2 | Activate Keyence system |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.3 | Perform startup check |  | Cable Fab Proc SU-1004-0963 |  | H |  |  |  |  |  |  |  |  |
| 5.4 | Produce startup sample(s) |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 5.5 | Examine major and minor edge facet sizes |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.6 | Perform cable surface inspection |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.7 | Measure and adjust cable parameters where necessary |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.0 | Qualification sample production |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Check CME with reference gauge block |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2 | Produce qualification sample |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3 | Measure cable parameters using CME |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4 | Verify cable lay pitch |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 6.5 | Measure residual twist |  | Cable Residual Twist Procedure SU-1007-5016 |  |  |  |  |  |  |  |  |  |  |
| 6.6 | Perform 10-stack measurement |  | Ten-stack Procedure SU-1007-5014 |  |  |  |  |  |  |  |  |  |  |
| 6.7 | Examine major and minor edge facet sizes |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.8 | Perform cable surface inspection |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.9 | Perform metallographic examination on cable transverse cross section |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 6.10 | This box is Intentionally left blank |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.11 | Approve qualification sample and cabling machine settings |  | Cable Fab Proc SU-1004-0963 |  | H |  |  |  |  |  |  |  |  |
| 7.0 | Cable production |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Check CME with reference gauge block |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2 | Produce unit length with inspection systems activated |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.0 | End of cable production |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | This box is Intentionally left blank |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2 | Register length fabricated during steady state production |  | Cable Fab Proc SU-1004-0963, Vector Traveler #464234 |  | N |  |  |  |  |  |  |  |  |
| 8.3 | Cut and label samples for QC and archive |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 9.0 | Release of cabling machine for next cable run |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1 | Re-measure tension of all strands |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2 | Archive leftover lengths |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.3 | Archive / discard leftover stainless steel core |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.4 | Unload wire spools |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.5 | Perform general inspection and clean-up / servicing of cabling machine |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.0 | Bare cable mechanical QC on Production Unit tail end piece |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Verify cable lay pitch |  | Cable Fab Proc SU-1004-0963 |  |  |  |  |  |  |  |  |  |  |
| 10.2 | Measure residual twist |  | Cable Residual Twist Procedure SU-1007-5016, Vector Traveler #464234 |  | N |  |  |  |  |  |  |  |  |
| 10.3 | Perform 10-stack measurement |  | Ten-stack Procedure SU-1007-5014 |  |  |  |  |  |  |  |  |  |  |
| 10.4 | Examine major and minor edge facet sizes |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.5 | Perform cable surface inspection |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.6 | Perform metallographic examination on cable transverse cross section |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.0 | Braiding insulation |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 | Send the completed bare cable to insulation vendor |  | Shipping Documents |  |  |  |  |  |  |  |  |  |  |
| 11.2 | Vendor braids insulation on the cable |  | US HiLumi Doc 75 |  |  |  |  |  |  |  |  |  |  |
| 11.3 | Vendor performs startup sample QC |  |  |  | R |  |  |  |  |  |  |  |  |
| 11.4 | Perform verification 10-stack measurement |  | Ten-stack Procedure SU-1007-5014, Vector Traveler #464342 |  | N/R |  |  |  |  |  |  |  |  |
| 11.5 | Vendor submits inspection report |  | US HiLumi Doc 75, Vector Traveler #464342 |  | N/R |  |  |  |  |  |  |  |  |
| 11.6 | Vendor sends the braided cable to FNAL/BNL/LBNL and informs LBNL tracking # |  | Relevant PO, Shipping Documents |  |  |  |  |  |  |  |  |  |  |
| 12.0 | Cable electrical QC (*) |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 | Perform heat treatment on at least 5 extracted strands |  | Vector Traveler #464342 |  | R |  |  |  |  |  |  |  |  |
| 12.2 | This box is Intentionally left blank |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.3 | Measure at least 5 extracted strands RRR |  | Vector Traveler #464342 |  | R |  |  |  |  |  |  |  |  |
| 13.0 | Production Unit approval |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 | Complete Vector Cable Fabrication Traveler |  | US HiLumi Doc 74, Vector Traveler #464234 |  | N/R |  |  |  |  |  |  |  |  |
| 13.2 | Complete Vector Cable QC Traveler |  | US HiLumi Doc 74, Vector Traveler #464342 |  | N/R |  |  |  |  |  |  |  |  |
| 13.3 | Complete Vector Insulated Cable QC Traveler |  | US HiLumi Doc 75, Vector Traveler #464342 |  | N/R |  |  |  |  |  |  |  |  |
| 13.4 | Coil winding team confirms braided cable receipt in good order |  |  |  | R |  |  |  |  |  |  |  |  |
| 13.5 | Approve Production Unit |  |  |  | H |  |  |  |  |  |  |  |  |

*Add as many rows as required

| NOTE: H = HOLD POINT N = NOTIFICATION POINT R = REVIEW AND APPROVAL OF REPORT W = WITNESS POINT | SUPPLIER: Approved by: Signature: Date: DD/MM/20YY | CLIENT: Approved by: Signature: Date: DD/MM/20YY | 3rd PARTY (if any): Approved by: not applicable Signature: Date: DD/MM/20YY |
| --- | --- | --- | --- |
| - N (Notification Point): CERN, or its authorized representative, is informed 5 working days in advance that a specific step has been completed and that the following step in the approved work-flow will be performed. A Notification Point does not affect the work-flow. Work can continue without CERN, or its authorized representative, reply. Data relative to the step are uploaded in MTF - H (Hold Point): CERN, or its authorized representative, is informed that a specific step has been completed. The work-flow is stopped until CERN, or its authorized representative, provides a Hold Point Clearance. The clearance is provided within 5 working days upon submission of the quality control documentation relative to the performed step. In case of clearance the work-flow can continue. In case of rejection, a recovery plan shall be discussed with CERN and submitted to CERN for final approval within 10 working days. - R (Review): The quality records will be reviewed. - W (Witness Point): CERN, or its authorized representative, intends to attend any specific step of the production. The supplier will notify the client with 10 working days in advance that the activity will be performed. |  |  |  |
| (*) Approved but requiring a second iteration on the acceptance tests for strand and cable |  |  |  |